Title: 		Robot Template Generetor User Manual (RTG User Manual)

Description:	Instructions on how to use the “Robot Template Generator” program for generating robot program templates for Epson, ABB, Kuka, Yamaha, and Kawasaki robots.

Created: 	16.09.2024.

Last update: 	16.09.2024.

By:		Luka Paladin

**************************************************************************************************

**Introduction**

Robot programming at Hidria utilizes a universal program scheme for all robot types.

This scheme defines how to initialize robot, set parameters, define I/O for communication with PLCs, performing homing, defining stations and movements in, out and between stations. All robot programs adhere to this scheme standardizing programming and communication with PLCs. (This scheme will be explained in a future manual.)

Since different robot manufactures use different software, instructions, and programing principales to make the robot program, the universal scheme must be expanded and adopted for specific manufactures. With this in mind the robot program templates are created for each unique manufacturer.

This program (Robot Template Generator) was created to simplify the start of programing a robot for its task. Utilizing user inserted information and a program template, a program is generated. This generated program contains the program structure (Main, init, homing, mainTask, testTask, goStation functions, MoveAway and MoveOnStataion), the neccessary functions (parameter definitions, signal resets, speed control functions, and other), signal files, points files, and all other neccesities.

The users task is then to properly implement the generated program into manufacturers specific software, look over the program, define additional points if neccessay, and define the movements. Running the testTask or testAll function the user can check the function of the program.

'

" ------- DOCUMENTATION FOR PROPER PROGRAMING OF YAMAHA ROBOTS -------

'

'   TEMPLATE FOR ROBOT PROGRAMING (YAMAHA)

'

'   TEMPLATE IS BASED ON PREVIOUSLY USED TEMPLATE FOR EPSON ROBOTS

'

'

"-- CHANGES MADE TO TEMPLATE FOR EPSON ROBOTS:

'

' > all "robot_go" functions have a "'define additional points" section to define

' above/outside/near station points in only one place before movement instructions.

' This point name has to be pre created in the robot points file. The point is then

' saved in the points file.  One point (ex. pAboveStation) can be used for

' multiple stations, but changed every time.

'

' > "from" and "to" signals (ex. "robot_from_home", "robot_to_station1") were changed

' from individual memory bits to 2 variables ("position_from" and "postion_to") which

' hold the station values defined in "ROBOT_INIT". Station values have to match point

' numbers in the point table. ex. P0 equals pHome so the value for constant "Home" is 0.

'

' > since Yamaha does not have a "power low" function, it is recreated by modifing the

' "TORQUE" value, as well as separate "SPEED" and a "power_mode" variable (0=low, 1=high).

'

' > O_PROGRAM_ERROR

'

' > O_BATTERY_LOW (set >Battery warning output< to ex.26 output number under Parameter -> IO -> BTALRM)

'

' > improved homing closest point search

'

' > "cycleTimer" function

'

' > "*armChange" function

'

'

'

'

" -- CAUTION WHEN PROGRAMING YAMAHA ROBOTS:

'

' > DO NOT modify "TOLE" value. It causes the robot to jolt/bounce when passing through

' some points when set too high. It is not healthy for the robot. Also it slows down

' program running, since program stops at steps where "TOLE" is modified.

'

' > using "CONT" function slows down robot movement significantly. To speed up movement

' "CONTPLS" has to be set high (>15000) which does not speed up the robot movement more

' than not using "CONT" at all!!! Also it creates unpredictable paths when going from

' linear to joint movement, and oposite. CONT can only be used for vertical linear moves.

'

' > not using "CONT" makes the robot stop at every point, so to prevent robot tool from

' jolting too much when using heavy loads with COG far from the robot flange it is

' recommended to increase "TOOL_WEIGHT" or lower "ROBOT_ACCEL" (max 60-70%).

'

' > vertical movemets (3rd axis) are faster when using "MOVE P" than "MOVE L". Robot has

' faster response when moving to and from a point. There is no reason to use "MOVE L".

'

' > parameter "DECEL" does NOT define the deceleration the same way as "ACCEL" does

' acceleration. "ACCEL" defines both acceleration and deceleration of a movement. "DECEL"

' defines the deceleration rate compared to the acceleration. So ACCEL=60 and DECEL=50

' means the acceleration is 60%, but deceleration is 50% of 60, so 30%. Use DECEL=100

' unless you want to have specificly slower deceleration than acceleration.

'

' > writing the "TORQUE" value slows down the program, This instruction waits for robot

' movement to stop. So only modify the values when necessery.

'

'

'

'

'--------------------------------------------------------------